# Posture Descriptor Engine — 18 Descriptors

Computed from the normalised 99-dim landmark vector (hip-centred, torso-scale-normalised). Used for step-completion matching and corrective feedback ranking; distinct from the MLP classifier.

| # | Descriptor Name | Description | Derived From (MediaPipe) |
| --- | --- | --- | --- |
|  | Joint Angle Descriptors (1–12) |
| 1 | leftElbowAngle | Angle at the left elbow joint (shoulder–elbow–wrist) | MediaPipe landmarks 11, 13, 15 |
| 2 | rightElbowAngle | Angle at the right elbow joint (shoulder–elbow–wrist) | MediaPipe landmarks 12, 14, 16 |
| 3 | leftKneeAngle | Angle at the left knee joint (hip–knee–ankle) | MediaPipe landmarks 23, 25, 27 |
| 4 | rightKneeAngle | Angle at the right knee joint (hip–knee–ankle) | MediaPipe landmarks 24, 26, 28 |
| 5 | leftHipAngle | Angle at the left hip joint (shoulder–hip–knee) | MediaPipe landmarks 11, 23, 25 |
| 6 | rightHipAngle | Angle at the right hip joint (shoulder–hip–knee) | MediaPipe landmarks 12, 24, 26 |
| 7 | leftShoulderAngle | Angle at the left shoulder (elbow–shoulder–hip) | MediaPipe landmarks 13, 11, 23 |
| 8 | rightShoulderAngle | Angle at the right shoulder (elbow–shoulder–hip) | MediaPipe landmarks 14, 12, 24 |
| 9 | leftAnkleAngle | Angle at the left ankle joint (knee–ankle–foot index) | MediaPipe landmarks 25, 27, 31 |
| 10 | rightAnkleAngle | Angle at the right ankle joint (knee–ankle–foot index) | MediaPipe landmarks 26, 28, 32 |
| 11 | spineTiltAngle | Angle between the torso vector (hip-mid → shoulder-mid) and the vertical axis (–Y); captures forward/backward lean | Shoulder midpoint vs. [0,–1,0] |
| 12 | shoulderLineTiltAngle | Angle between the left→right shoulder vector and the horizontal axis; captures lateral lean and twist | Landmarks 11, 12 vs. [1,0,0] |
|  | Spatial / Distance Descriptors (13–18) |
| 13 | footSeparation | Euclidean distance between left and right ankles (in normalised landmark units); indicates stance width | Landmarks 27, 28 |
| 14 | kneeSeparation | Euclidean distance between left and right knees; indicates leg spread | Landmarks 25, 26 |
| 15 | leftWristHeight | Y-coordinate of the left wrist (negative = above hips in hip-centred space); captures arm raise | Landmark 15, Y component |
| 16 | rightWristHeight | Y-coordinate of the right wrist; captures arm raise on the right side | Landmark 16, Y component |
| 17 | leftWristShoulderDist | Euclidean distance from the left wrist to the left shoulder; captures arm extension | Landmarks 15, 11 |
| 18 | rightWristShoulderDist | Euclidean distance from the right wrist to the right shoulder; captures arm extension | Landmarks 16, 12 |

All angles in radians (0–π). All distances in normalised landmark units (1 unit ≈ torso length). Each descriptor is compared against per-step reference statistics (median ± MAD) to produce a z-score; the top-2 highest |z| descriptors become the corrective feedback phrases.

# Ollama LLM Fallback — Prompt Design

Triggered after 7 seconds of continuous posture violation. Runs fully offline against a local Ollama server (no cloud dependency). Two modes: text (landmark coordinates) and vision (camera frame).

| Field | Content |
| --- | --- |
| Model | llama3.2 (landmark/text mode) · llava / moondream (vision mode, when camera frame provided) |
| Trigger | User has been in posture violation for ≥ 7 seconds with no step advancement |
| System Role | "You are an expert yoga instructor and certified biomechanics analyst. A student is practicing '{poseName}'." |
| Age-adaptive guidance | Under 40: energetic, challenge-oriented tone — encourage fuller range of motion. 40+: gentle, safety-first tone — prioritise joint safety, offer easier modifications. |
| Landmark prompt (text mode) | Step to verify is injected. 33 MediaPipe landmarks (x, y, z) are appended as JSON. Key index map provided: Nose=0, Shoulders=11/12, Elbows=13/14, Wrists=15/16, Hips=23/24, Knees=25/26, Ankles=27/28. |
| Vision prompt (vision mode) | Actual camera frame (base64 image) sent instead of coordinates. Model reasons about visible body position directly. |
| Output format | Raw JSON only — no markdown: { "stepComplete": true/false, "feedback": "1–2 concise sentences on what to fix, or an encouraging remark if correct." } |
| Rules | Be reasonably generous — if pose roughly matches, mark stepComplete: true. Feedback must describe what is actually observed. Maximum 2 sentences. |